Synthetic Data Generation – Governing Physics and Variables

Centrifugal Pump Case

1. Scope and Objective

The synthetic dataset is generated using a physics-based representation of a single-phase centrifugal pump system operating under steady-state conditions. The objective is to produce physically consistent operating points by varying key controllable variables and solving the coupled pump–system equations.

The model captures the fundamental relationships between pump speed, flow rate, system resistance, fluid properties, and pump performance.

2. Manipulated (Independent) Variables

Each synthetic operating point is generated by sampling the following independent variables within prescribed ranges:

| Variable | Description |
| --- | --- |
| Ps | Suction pressure |
| N | Pump rotational speed |
| T | Fluid temperature |
| ρ | Fluid density |
| μ | Fluid dynamic viscosity |
| Hstatic | System static head |
| k | System friction coefficient |

These variables define the operating envelope over which the pump–system interaction is evaluated.

3. Pump Performance Model

The centrifugal pump is represented by a head–flow curve defined at a reference speed:

Hbase=f(Q)

Pump head at arbitrary speed is computed using the centrifugal pump affinity laws:

H(Q,N)=Hbase(Q)NNbase2

where:

- Qis the volumetric flow rate

- Nis the pump rotational speed

- Nbaseis the reference speed at which the pump curve is defined

4. System Curve Model

The system hydraulic resistance is modeled using a quadratic system curve:

Hsystem(Q)=Hstatic+kQ2

where:

- Hstaticrepresents elevation and static pressure effects

- krepresents lumped frictional losses

Optional Viscosity Correction

To account for changes in fluid viscosity, the friction coefficient may be scaled as:

keff=kμμrefα

where:

- μis the sampled fluid viscosity

- μrefis a reference viscosity

- αis an empirical exponent

5. Operating Point Determination

For each synthetic case, the steady-state operating point is determined by solving for the flow rate at which pump head equals system head:

H(Q,N)=Hsystem(Q)

The operating flow rate is defined as:

Qop=arg⁡min⁡Q∣H(Q,N)-Hsystem(Q)∣

This represents the equilibrium condition of the pump–system interaction.

6. Pump Power Model

Pump shaft power is computed from a reference power–flow relationship and scaled using affinity laws:

Ppump(Q,N)=Pbase(Q)NNbase3

where:

- Pbase(Q)is the pump power at reference speed

- Power scaling reflects the cubic dependence on rotational speed

7. Pressure Relationship

Discharge pressure is obtained from suction pressure and pump head:

Pd=Ps+ρgH(Q,N)

where:

- Psis suction pressure

- Pdis discharge pressure

- ρis fluid density

- gis gravitational acceleration

(Pressure rise across the pump is therefore implicitly determined and is not treated as an independent variable.)

8. Summary of Physical Assumptions

- Steady-state operation

- Single-phase liquid flow

- No cavitation or vapor formation

- Pump and system curves are smooth and continuous

- Thermal effects are represented only through density and viscosity variation

9. Resulting Outputs

For each operating point, the model produces physically consistent values of:

- Flow rate

- Pump power

These outputs form the basis of the synthetic dataset used for downstream modeling and analysis.